This document describes how to stop NNM’s netmon and APA processes from changing the management address they use.  It also describes how to correct devices that have the “wrong” address.

Cisco devices (and most other vendor’s routers) already support a “loopback” address that is essentially a “virtual” address that works as long as there is a physical interface that is running.  I.e., if the physical interface associated with the loopback fails, then the router will look for another functioning interface and associate it with the loopback address.  Thus the loopback address always stays the same.

Please do not confuse Cisco and router “loopback” from Unix computer loopback that does not go out the stack.  Unix uses this for example for interprocess communication.

Since Cisco and other routers have this functionality, why have NNM try to find another management address?  Again, this document describes how to shut off NNM’s netmon and APA processes from changing the management address they use.  After doing this, you’ll be relying on Cisco’s loopback functionality.

After shutting off address changing in NNM, if there are only a few devices with incorrect addresses, then use the topodbpoke tool.  Ask HP Software support for this special tool.  Be careful, the poke tool can poke something that should not be poked!  The topodbpoke tool is unsupported and you use it at your own risk.

If there are more than a few “incorrect” addresses, then there will be too much poking, and more poking means more chances to accidentally poke the wrong thing.  It’s definitely easiest to remove the netmon topology database (please backup before doing this) and have netmon rediscover the network.  If you want to control this you can use loadhosts, or the other alternative is to use a seedfile with autodiscovery shut off.  I.e., with the loadhosts or seedfile lists you have much more control over addresses.

If you have netmon autodiscovery on, it’s important you let the netmon database stabilize over a few days (not hours) before you do another ET discovery.

Now that you have a new netmon and ET database, look for devices with SNMP management addresses that you don’t like and poke them.

At this point, you are relying on Cisco’s loopback functionality, and not having NNM do this.  This has been very successful at a number of large customers that use Cisco.

# How to stop NNM’s netmon and APA processes from changing the management address

0. ovstop -c

1. First we must disable the "switching feature".  We’ll do netmon first.  Edit the file $OV_LRF/netmon.lrf.

2. Add the argument "-k adjustNodeSnmpAddr=false" right after the "-k migrateHsrpVirtualIP=true" portion.  It should look something like this.  This is all one line.

OVs_YES_START:ovtopmd,pmd,ovwdb:-P -k segRedux=true -k migrateHsrpVirtualIP=true -k adjustNodeSnmpAddr=false:OVs_WELL_BEHAVED:15:PAUSE

3. Save the file and run

ovaddobj $OV_LRF/netmon.lrf

4. Now we need to shut it off in APA.  Edit /etc/opt/OV/share/conf/nnmet/paConfig.xml

Find the following section and change the value from false to true and save the file.

<parameter>

<name>MgmtAddrInhibited</name>

<title>Management Address Inhibited</title>

<description>

This address object parameter is used to tune the APA

pickManagementAddress functionality.  For a given address,

if this parameter is true, the address will never be used

as a management address if the current management address

fails.

</description>

<varValue>

<varType>Bool</varType>

<value>true</value>

</varValue>

</parameter>

Now your management addresses shouldn’t change.  All of these changes are “sticky” and you won’t have to do them again.

# How to change “incorrect” management addresses

After shutting off address changing in NNM, if there are only a few (say less than 10) devices with incorrect addresses, then use the topodbpoke tool (details below).  Ask HP Software support for this special tool.  Be careful, the poke tool can poke something that should not be poked!  The topodbpoke tool is unsupported and you use it at your own risk.

If there are more than a few “incorrect” addresses, then there will be too much poking, and more poking means more chances to accidentally poke the wrong thing.  It’s definitely easiest to remove the netmon topology database and have netmon rediscover the network.

First backup your system, then

ovstop -c

cd $OV_DB/nnmet

rm -rf */*

Now restart netmon discovery.  If you want to control netmon you can use loadhosts, or the other alternative is to use a seedfile with autodiscovery shut off.  I.e., with the loadhosts or seedfile lists you have much more control over addresses.

Now Rerun ET discovery (etrestart.ovpl -verbose -disco)

Bring up a Node Details page and you'll see the new preferred management address.  Make sure it's what you want.

Now restart APA (ovstop -c ovet_poll; ovstart -c ovet_poll)

----------------------------

Last, you can use the topodbpoke command. Again, ask HP Software support for this special tool.  The topodbpoke tool is unsupported and you use it at your own risk.

Now we must switch back your wrong address to the loopback addresses.  You’ll need to identify these nodes using the following steps:

1. Find a node that has the wrong management address.  I usually do this by double clicking on the node in a dynamic view.  You can also get good node info by running "ovtopodump <IP or nodename>"

2. Note the NNM Object ID from the ovtopodump.  Here's an example.  2767 is the NNM Object ID.

OBJECT ID(S)        OBJECT                 STATUS      IP ADDRESS
  2767        IP    cisco6509.all.fcov     Marginal    15.2.128.1     
  2767/2628   IP    cisco6509.all.fcov     Normal      15.2.128.1     
  2767/2629   IP    cisco6509.all.fcov     Critical    10.16.88.2     
  2767/2630   IP    cisco6509.all.fcov     Normal      10.162.237.4   
  2767/2631   IP    cisco6509.all.fcov     Normal      15.2.121.254   
  2767/2632   IP    cisco6509.all.fcov     Normal      15.2.128.9     
  2767/2633   IP    cisco6509.all.fcov     Normal      15.2.131.50

3. Decide what the management address should be.

4. Copy topodbpoke to the management station.  Be sure to use the correct topodbpoke tool that matches the OS of the management station.  Make sure topodbpoke is executable.  For example, on Unix do a "chmod +x topodbpoke" first.

5. Use this tool to change the management address.  Here is the usage:

topodbpoke -nodeSNMP <IP-addr> <NNM-object-ID>

So for our example, let’s change the management address to 15.2.128.9.  We’ll run

topodbpoke –nodeSNMP 15.2.128.9 2767

You should see results like:

# ./topodbpoke -nodeSNMP 15.2.128.9 2767

DEBUG: 1170286516: Starting topology load.

DEBUG: 1170286516: Loading TopoInfo

DEBUG: 1170286516: Loading Stations

DEBUG: 1170286516: Loading Networks

DEBUG: 1170286516: Loading Segments

DEBUG: 1170286516: Loading Nodes

DEBUG: 1170286516: Inserting Nodes

DEBUG: 1170286517: Checking Topo

DEBUG: 1170286517: Done.

Found object cisco6509.all.fcov (oid = 2767)

Updated node SNMP address for cisco6509.all.fcov (oid = 2767)

6. Now run ovtopodump to check the new value:

ovtopodump 2767

OBJECT ID(S)        OBJECT                 STATUS      IP ADDRESS

2767        IP    cisco6509.all.fcov     Marginal    15.2.128.9

2767/2628   IP    cisco6509.all.fcov     Normal      15.2.128.1

2767/2629   IP    cisco6509.all.fcov     Critical    10.16.88.2

2767/2630   IP    cisco6509.all.fcov     Normal      10.162.237.4

2767/2631   IP    cisco6509.all.fcov     Normal      15.2.121.254

2767/2632   IP    cisco6509.all.fcov     Normal      15.2.128.9

2767/2633   IP    cisco6509.all.fcov     Normal      15.2.131.50

After you have completed this for any nodes that have changed, you can re-run discovery.    First run

ovstart –c

etrestart.ovpl –verbose -disco